## Lighting SWL 								BU 2026

The purpose of this document is to demonstrate the specific weights of lamps and modifiers combined to allow you to select an appropriate stand based on the SWL (Safe Working Load) rating of the stand. At no point should any lighting setup be used that exceeds the SWL rating of the stand in use.

To calculate which stand you require to maintain a safe working approach please combine the weights of fixture/modifiers/overhead grip that you are planning to use, then select an appropriate stand based on these calculations.

Fixture weight + Modifier weights + additional items = total load

When selecting a stand please ensure to leave headroom for batteries as these can weigh up to 1.3kg each. This is relevant for all Aputure lamps due to the control box being mounted on the stand.

Please also remember to book sandbags for counter weighting and additionally safety chains when lighting is being used at height.

Lamps & Modifiers

| Fixture Name | Total weight including ballast/control unit. | Appropriate Modifiers | Weight |
| --- | --- | --- | --- |
| Amaran 200X | 2.62g | Aputure 2X Fresnel Aputure LS Barn Doors Aputure Lantern Aputure Lantern 90 Aputure LightBox 30x120 Aputure Light Dome II Aputure Spotlight 26° | 1.24kg 1.04kg 1.30kg 2.59kg 1.08kg 1.28kg 5.11kg |
| Aputure 300X | 5.73kg | Aputure 2X Fresnel Aputure LS Barn Doors Aputure Lantern Aputure Lantern 90 Aputure Lightbox 30x120 Aputure Light Dome II Aputure Spotlight 26° | 1.24kg 1.04kg 1.30kg 2.59kg 1.08kg 1.28kg 5.11kg |
| Aputure 600D / X | 10.44kg | Aputure F10 Fresnel Aputure F10 Barn Doors Aputure F10 Barn Door Leaves Aputure Lantern Aputure Lantern 90 Aputure Lightbox 30x120 Aputure Light Dome II Aputure Light Dome 150 | 3.85kg 3.70kg 2.65kg 1.30kg 2.59kg 1.08kg 1.28kg 3.32kg |
| Aputure Nova P300C | 10.78kg | Aputure Nova P300C Softbox | 1.35kg |
| Aputure Nova P600C | 14.17kg | Octa 7' SBRO + Rabbit Ears | 6.5kg |
| Arri HMI 575w | 6.75kg | n/a |  |
| Arri HMI 1.25kw | 11.60kg | n/a |  |
| Amaran F21C | 3.00kg | n/a |  |
| Amaran F22C | 4.03kg | n/a |  |
| Amaran F22X | 3.85kg | n/a |  |
| Lishuai LED Soft Light C-700RSV | 3.10kg |  |  |
| Lishuai Led Square Soft Light C-518ASV | 2.08kg |  |  |
| Astra 6X LED Panel | 3.20kg |  |  |
| Kino Flo L20 | 6.50kg |  |  |
| Kino Flo L30 | 8.00kg |  |  |

Overhead Grip

| Matthews Mini Boom | 3.40kg | SWL kg at Minimum Extension 3.6kg at Maximum Extension |
| --- | --- | --- |
| Avenger Junior Boom | 14.80kg | SWL 40kg at Minimum Extension 10.0kg at Maximum Extension |

Batteries

| Swit S-8087S V-Lock Battery | 0.73kg |
| --- | --- |
| Swit S-8132S V-Lock Battery | 1.00kg |
| Swit PB-S986 V-Lock Battery | 0.83kg |
| IDX CUE-D95 V-Lock Battery | 0.85kg |
| Anton Bauer 150 V-Lock Battery | 1.26kg |
| Hawkwood’s RP-200HP Battery (Aputure 600D/X only) | 1.21kg |

Stands SWL Rating (Maximum Weight Limit)

| Manfrotto Mini 1051BAC | 4.0kg |
| --- | --- |
| Manfrotto 1004BAC | 9.0kg |
| Avenger C-Stand | 10.0kg |
| Manfrotto Wind-up Stand | 30.0kg |
| Combo Stand | 40.0kg |
| Wheeled Combo Stand | 40.0kg |